FLEXLOC SPECIFICATIONS
Effective March 17,1999

For Technical Assistance regarding UL and the requirements involved to meet UL or other codes, please contact MBCI Technical Services at 877-713-6224, ext. 28612.  If you have difficulties with downloading or editing these specifications, contact our Help Desk at info@mbci.com.

Specifier:  Notation (#) means that text following is a specifier's note or sample.

## PART 1 - GENERAL

1.01 DESCRIPTION

# Specifier: Do not alter paragraph A except by adding section title in brackets.

A. General:

1. Furnish all labor, material, tools, equipment and services for all preformed [# Choose one: soffit, fascia, curved arches] as indicated, in accordance with provisions of Contract Documents.

2. Completely coordinate with work of all other trades.

3. Although such work is not specifically indicated, furnish and install all supplementary or miscellaneous items, appurtenances and devices incidental to or necessary for a sound, secure and complete installation.

4. See Division 1 for General Requirements.

B. Related work specified elsewhere:

1. Structural steel: Section 05100.

2. Steel joists: Section 05200 or 05400.

3. Flashing and sheet metal: Section 07600.

# Specifier: Delete references to sections not used and add any references which become pertinent.

1.02 QUALITY ASSURANCE

A. Applicable standards:

1. SMACNA: "Architectural Sheet Metal Manual" Sheet Metal and Air Conditioning Contractors National Association, Inc.

2. AISC: "Steel Construction Manual" American Institute of Steel Construction.

3. AISI: "Cold Form Steel Design Manual," American Iron and Steel Institute.

4. ASTM A792-83-AZ50: Specifications for steel sheet, aluminum-zinc alloy coated (galvanized) by the hot dip process, general requirements (Galvalume).

B. Manufacturer's qualifications:

1. Manufacturer has a minimum of three years' experience in manufacturing metal roof systems of this nature.  Panels specified in this section shall be produced in a factory environment (not job site roll formed) with fixed-base roll forming equipment assuring the highest level of quality control.  A letter from the manufacturer certifying compliance will accompany the product material submittals.

C. Installation contractor's qualifications:

1. Installation contractor shall be an approved installer, certified by the manufacturer within one year of the beginning of installation of the metal roof system, specifically for MBCI's FlexLoc metal roof system and meet the following minimum criteria:

a.  Maintain $250,000 minimum general liability insurance coverage.

b. Maintain statutory limits of worker's compensation coverage as mandated by law.

c. Has no viable claims pending regarding negligent acts or defective workmanship on previously performed or current projects.

d. Has not filed for protection from creditors under any state or federal insolvency or debtor relief statutes or codes.

e. Project foreman is the person having received specific training in the proper installation of the specified metal roof system and will be present to supervise whenever material is being installed.  Specific training program shall include the following:

1. The instructor must have a minimum of 10 years' experience in the application of metal roof systems.

2. A formal syllabus for the classroom and hands-on training.

3. Classroom instruction with review and thorough understanding of the specific product's technical manual.

4. Hands-on mock-up instruction with review and thorough understanding of the specific product's details.

5. The installation contractor must pass a written and oral exam.

f. Provide five references from five different architects or building owners for projects that have been in service for a minimum of two years, stating satisfactory performance by the installation contractor.

g. Provide certification letter that installation contractor has a minimum of three years' of metal product installation experience immediately preceding the date upon which work is to commence.

D. Installation quality control:

1. The general contractor shall provide a third party metal roof consultant, working for the general contractor and approved by the metal roof system manufacturer, to approve the metal roof system installation drawings and inspect the installation of the FlexLoc metal roof system at the following stages of the metal roof system installation:

a. At completion of the first assembly of secondary framing.

b. At the first assembly of substrates.

c. At completion of the first assembly of panels.

d. Prior to installing a typical trim/flashing component.

e. Prior to installing an unusual component or condition.

f. At final completion of all metal roof system work.

2.The third party metal roof consultant shall provide written and photographic reports, to be submitted to the architect (owner), metal roof system manufacturer, metal roof system installation contractor and general contractor, appraising the installation of the metal roof system at each of the project progress stages.  The installation contractor shall make all necessary corrections, additions or remedial actions to resolve any issues raised in the reports.

3. The third party metal roof consultant shall have the authority to have any and all metal roof system work corrected, as required, to insure the proper installation and weathertightness of the metal roof system.

1.03 SUBMITTALS

A. Shop drawings:

1. Submit complete shop drawings and erection details, approved by the general contractor's third party metal roof consultant, to the architect (owner) for review. Do not proceed with manufacture prior to review of shop drawings. Do not use drawings prepared by the architect (owner) for shop or erection drawings.

2. Shop drawings show methods of erection, elevations and plans of roof and wall panels, sections and details, anticipated loads, flashings, roof curbs, vents, sealants, interfaces with all materials not supplied and proposed identification of component parts and their finishes.

B. Samples:

1. Submit samples and color chips for all proposed finishes.

a. Submit one 12 in. long sample of panel, including clips.

b. Submit two 3 in. x 5 in. color chip samples in color selected by the architect (owner).

C. Warranty(s):

1. Metal roof system manufacturer, upon final acceptance for project, furnish a warranty covering bare metal against rupture, structural failure and perforation due to normal atmospheric corrosion exposure for a period of 20 years.

2. Covering paint finish against cracking, checking, blistering, peeling, flaking, chipping, chalking and fading for a period of [# Choose one: twenty (20) years for roof panels (premium thermoset silicone polyester) or twenty (20) years for wall panels and twenty (20) years for roof panels (premium fluorocarbon coating produced with Kynar 500 or Hylar 5000 resin)].

D. Metal roof system fabrication certification:

1. Submit a letter from the metal roof system manufacturer certifying the FlexLoc panels have been produced in a factory environment (not job site roll formed) with fixed-base roll forming equipment.

E. Third party metal roof consultant approval:

1. Submit a letter from the metal roof system manufacturer indicating acceptance of the general contractor's third party metal roofing consultant for use on this specific project.

F. Installation contractor's qualification:

1. Submit certificate from manufacturer certifying that installer of the metal roof system has met all of the criteria outlined in "1.02 C. Installer's qualifications" and is an authorized installer certified by the manufacturer within one year of the beginning of installation of the metal roof system.

2. Submit the formal syllabus for the classroom and hands-on training.

3. Submit five references from five different architects or building owners for projects that have been in service for a minimum of two years, stating satisfactory performance by the installation contractor.

G. Metal roof system installation inspection reports:

1. Submit written and photographic metal roof system installation inspection reports from the general contractor's third party metal roof consultant appraising the installation of the metal roof system.  The written and photographic inspection reports are to be submitted to the architect (owner), metal roof system manufacturer, metal roof system installation contractor and general contractor.

2. A separate report is to be submitted for each of the following stages of the metal roof system installation:

a. At completion of the first assembly of secondary framing.

b. At the first assembly of substrates.

c. At completion of the first assembly of panels.

d. Prior to installing a typical trim/flashing component.

e. Prior to installing an unusual component or condition.

f. At final completion of all metal roof system work.

1.04 PRODUCT DELIVERY, STORAGE AND HANDLING

A. Delivery:

1. Deliver metal roof system to job site properly packaged to provide protection against transportation damage.

B. Handling:

1. Exercise extreme care in unloading, storing and erecting metal roof system to prevent bending, warping, twisting and surface damage.

C. Storage:

1. Store all material and accessories above ground on well skidded platforms. Store under waterproof covering. Provide proper ventilation of metal roof system to prevent condensation build-up between each panel or trim/flashing component.

PART 2 - PRODUCTS  [# FLEXLOC; for soffit, fascia, curved arches]

2.01 MATERIALS

A. Metal roof system profile:

1. 9½ inch wide

B. Metal roof system style:

1. Flush face.

2. Concealed fastener.

C. Gauge: [# choose one]

1. 22 gauge.

2. 24 gauge.

3. 26 gauge.

D. Substrate:

1. Galvalume steel sheet, 0.5 ounces/square foot, minimum yield 50,000 PSI.

E. Texture: [# choose one]

1. Smooth.

2. Embossed (reduces oil canning effect).

F. Finish: [# choose one]

1. Premium thermoset silicone polyester (20 year warranty).

2. Premium fluorocarbon coating produced with Kynar 500 or Hylar 5000 resin. (20 year warranty).

3. Bare Galvalume (20 year warranty).

G. Color:

1.Selected from metal roof system manufacturer's standard offering.

H. Acceptable manufacturer:

- MBCI - Houston, TX - (281) 445-8555

- MBCI - Adel, GA - (229) 896-3272

- MBCI - Atlanta, GA - (770) 948-7568

- MBCI - Atwater, CA - (801) 530-4975

- MBCI - Ennis, TX - (972) 875-6586

- MBCI - Indianapolis, IN - (317) 398-4400

- MBCI - Lubbock, TX - (806) 747-4291

- MBCI - Memphis, TN - (601) 429-4009

- MBCI - Oklahoma City, OK - (405) 672-7676

- MBCI - Phoenix, AZ – (623) 907-4616

- MBCI - Rome, NY - (315) 339-9701

- MBCI - San Antonio, TX - (210) 661-2409

- MBCI - Salt Lake City, UT - (801) 973-0911

# Delete: MBCI plant locations that do not apply.

I. Optional manufacturers:

1. Berridge Manufacturing - Houston, TX.

# Insert: Architect's (owner's) method of approval of "or equals."

J. Other manufacturers desiring approval, comply with Section 01630.

2.02 FABRICATION

A.  Material shall be in-line tension leveled prior to roll forming finished panel profile.

B.  Roll form panels in continuous lengths, full length of detailed runs.

C.  Standard panel length shall be no more than 45 feet long (for longer length availability, contact manufacturer).

D.  Fabricate trim/flashing and accessories to detailed profiles.

E.  Fabricate trim/flashing from same material as panel.

## PART 3 - EXECUTION

3.01 SURFACE CONDITIONS

A. Examination:

1. Inspect installed work of other trades and verify that such work is complete to a point where this work may continue.

2. Verify that installation may be made in accordance with approved shop drawings and manufacturer's instructions.

B. Discrepancies:

1. In event of discrepancy, notify the architect (owner).

2. Do not proceed with installation until discrepancies have been resolved.

3.02 INSTALLATION

A.  Install metal roof system so that it is weathertight, without waves, warps, buckles, fastening stresses or distortion, allowing for expansion and contraction.

B.  Install metal roof system in accordance with manufacturer's instructions and shop drawings.

C.  Provide concealed anchors at all panel attachment locations.

D.  Install panels plumb, level and straight with seams parallel, conforming to design as indicated.

3.03 CLEANING, PROTECTION

A.  Dispose of excess materials and remove debris from site.

B.  Clean work in accordance with manufacturer's recommendations.

C.  Protect work against damage until final acceptance. Replace or repair to the satisfaction of the architect (owner), any work that becomes damaged prior to final acceptance.

D.  Touch up minor scratches and abrasions.

# END OF SECTION

DISCLAIMER: MBCI makes no warranty, express or implied, as to the merchantability or fitness for any particular purpose of any product manufactured by an optional manufacturer. If you choose to use a product manufactured by an optional manufacturer, as defined herein, you take the product as is and at your own risk.  Descriptions and specifications contained herein were in effect at the time this publication was approved for printing.  MBCI reserves the right to discontinue products at any time or change specifications and/or designs without notice and without incurring obligation.