Engineering Technical Bulletin

No. 414-02-21                                                                                  July 21, 2017

Susceptibility to Leakage Test for Roof System Assemblies

PBR Panel

PRODUCT:  MBCI PBR Roof System.

MBCI’s Min. 24 ga. PBR Roof panel with HW-507 ½” x 3/32” tape sealer in the side

laps and HW-504 Tri-Bead tape sealer in the end laps.

TEST PROCEDURES: FM 4471 Appendix G (Standard TAS 114 Appendix G)

The FM susceptibility to Leakage Test procedure are designed to assess the

potential for water penetrate a roof panel fabricated with a typical side lap and, if

applicable, a typical end lap.

Test Results:

| Test | Water Spray Leakage Test | Static Water Leakage Test |
| --- | --- | --- |
|  | 5 GAL/SF/HR for 15 Mins. | 6" Water depth for 7 days |
| PBR Panel | No signs of water leakage during the test period and 15 minutes after. | Passed - No Leakage |

Test Report: T124-08, dated February 12, 2008 by Farabaugh Engineering and Testing Inc.

- Material Subject to Change Without Notice	 	MBCI Proprietary Information